A guideline to generate stochastic earthquake scenarios for tsunami modelling 
[the R-PTHA code version]

Prepared by Ryan (29 October 2020)

In this tutorial, I will give you a tutorial on how I prepare a set of stochastic scenarios using the PTHA code from Geoscience Australia. The code is freely available from this link: https://github.com/GeoscienceAustralia/ptha. You can follow the guideline given to compile and build the code.

There are three main steps:

- Subfaults preparation

- Generate unit sources displacement

- Generate stochastic scenarios

I use those three steps above to generate a set of tsunami Greens Functions on the JAGURS code, which is also freely available from this link: https://github.com/jagurs-admin/jagurs. This Greens Function would be useful for analysing off-shore tsunami hazard from thousands scenarios. Alternatively, you can just run forward modelling for all (or selected) the stochastic scenarios. Compiling JAGURS is a little bit tricky due to the libraries requirement. I will show you on how I compiled JAGURS on my Ubuntu Mate 20.04 machine at another chance.

Software and/or program needed:

- R- for the PTHA code

- QGIS

Let’s do it!

For this tutorial, I will use the Makassar Thrust Fault from the central segment (Irsyam et al., 2020) as an example.

General preparation

- Create a working directory (e.g. MakassarStraits). Inside this, create scripts directory to store scripts needed.

- Inside scripts you will need:

  - 01_run_convert_traces_to_contours.R

  - 02_make_initial_downdip_lines.R

  - 03_produce_unit_sources.R

  - 04a_stochastic_tsunami_sources.R

  - collate_source_traces.R

- Compile the PTHA code inside the scripts directory.

- If you already have rptha_X.X.XXX.tar.gz from others, you could just do command below from your terminal:

  - $R CMD INSTALL rptha_X.X.XXX.tar.gz

- When it finished without error, check whether the code is successfully compiled.

  - $R <enter>

  - >>library(rptha) <enter>

  - If you got something like above, it means no problem so far.

Subfaults preparation

- Create a fault contours shapefile (if you don’t have it)

- Create fault_traces directory inside MakassarStraits

- Inside fault_traces you need MksThrust_Central.shp as follows

- The MksThrust_Central.shp at least should have attribute table such as on the right figure.

- Edit 01_run_convert_traces_to_contours.R; important variables:

  - sources_traces = c(“../fault_traces/MksThrust_Central.shp”) << location of the fault_traces shp

  - output_dir = “../contours”

- Run the script from terminal:

  - $Rscript 01_run_convert_traces_to_contours.R <enter>

  - If there is no error, you will have a new directory: contours inside MakssarStraits, and MksThrust_Central_contours.shp

- Open the contours.shp on QGIS. You will have a slightly dodgy contours at the first contour (shown below). You may edit it manually.

- You may edit the contours manually to example below. The contour.shp needs attribute table of “level” in string format. Alternatively, if you have contour.shp from other sources (e.g. SLAB2.0), you can directly use it and adapt the format as needed.

- Create down-dip lines to approximately create the subfaults

- Edit 02_make_initial_downdip_lines.R:

  - source_shapefile = “../contours/MksThrust_Central_contours.shp”

  - out_shapefile = “../downdip_lines/MksThrust_central_downdip.shp”

  - desired_unit_source_length = 40 << to set up the subfault length (in km) along the strike

- Run the script from terminal:

  - Create downdip_lines directory inside MakassarStraits

  - $Rscript 02_make_initial_downdip_lines.R <enter>

  - If no error, you will have MksThrust_Central_downdip.shp inside downdip_lines

- Open MksThrust_Central_downdip.shp and manually edit as needed

  - If you don’t like the subfaults, you can:

    - repeat step 1 and 2

    - I recommend to manually edit the contours.shp and regenerate the downdip_lines.shp

Generate unit sources displacement

In this step, you will generate earthquake deformation from each subfaults according to the Okada (1985) formula.

- Create outputs directory

- Edit 03_produce_unit_sources.R:

  - all_sourcezone_shapefiles = Sys.glob('../contours/*.shp')

  - all_sourcezone_downdip_shapefiles = Sys.glob('../downdip_lines/*.shp')

  - desired_subfault_length = 40

  - desired_subfault_width = 20

  - sourcezone_rake = rep(90,...) << change 90 to -90 if you need a normal mechanism, or as needed

  - tsunami_source_cellsize = 0.01 #in degree

  - output_base_dir = “../outputs/

  - Other variables can read from the comments or study all the scripts :)

- Run the script from terminal:

  - $Rscript 03_produce_unit_sources.R 1 << change 1 into number of faults if there are multi-sources.

- If no error, you will have new directories and all_discretized_sources.RDS inside outputs:

  - Unit_source_data/MksThrust_Central_contours/

    - Inside this, you will have a bunch of .tif and .RDS files.

    - The .tif file is earthquake displacement for each subfault.

    - In this case, you will have 10 subfaults: 5x along strike, and 2x along dip

    - MksThrust_Central_contours_<row number along dip>_<column number along strike>.tif

  - unit_source_grid

    - Polygon of the subfaults

  - all_discretized_sources.RDS

- In the final run, you may need to increase the tsunami_source_cellsize and change the desired_subfault_length and desired_subfault_width.

Generate stochastic scenarios

- Create stochastic directory

- Edit 04a_combine_tsunami_sources.R. This is a simplification version from the original example to be able export the slip distribution.

  - unit_source_dirname = “../outputs/Unit_source_data” <.tif files location>

  - sourcename = “MksThrust_Central_contours”

  - desired_Mw = 7.7 <random as needed>

  - target_location = c(lon,lat) <you can fix the peak slip location or put it randomly>

  - number_of_sffm = 100 <number of scenarios you want to generate>

  - You can edit between line 64 and 76. There are several options for the relation (earthquake scaling)

- Run the script from terminal:

  - $Rscript 04a_stochastic_tsunami_sources.R

- If no error, you will have a bunch of .csv files inside stochastic directory.

  - Slip distribution:

      - It shows slip (in metre) for #row and #column (A,B,etc.) of the subfault

  - subfaults_MksThrust_Central_contours.csv

    - Subfaults configuration:

- Every time you do this step, you will get different results even using the same parameters (or you didn’t change the script)

Additional

Depends on your needs and the final goal, after this you can do whatever you want:

- You can convert all the .tif files into a .grd file as needed for the JAGURS then generate tsunami Greens Function. Then use the stochastic scenarios to analyse off-shore tsunami heights. Even, you can continue doing a PTHA.

- Alternatively, by combining the .tif files and the slip distribution from each stochastic scenarios, you can generate a total earthquake displacement (e.g. in grd file) then use it as initial condition for your tsunami modelling.

- TotalDisplacement = i=1Nsubfaultunit_source[i]×slip[i]

Here how I do the b) option on Python. It’s a “dirty” script.

- Create ori_displacement_grid inside outputs directory

- Create scenarios_grid inside stochastic

- Edit combine_unitsources_and_slip.py

- Run combine_unitsources_and_slip.py

- You will have grd files inside scenarios_grid, then continue as needed.